Mobile phone base stations and cellular structure

These questions use information about frequency allocation for mobile phone service providers available at http://www.ero.dk/gsm

1. The information is sent from the base station to the mobile phone on the uplink band of 890MHz – 915MHz for GSM. What are the uplink frequencies for Vodaphone?

……………………………………………………………………………………………

2. The information is sent from the mobile phone to the base station on the downlink band of 935MHz – 960MHz for GSM . What is the total downlink bandwidth for vodaphone?

………………………………………………………………………………………….

3. Each mobile telephone channel has a bandwidth of 200kHz. How many downlink channels does Vodaphone have?

…………………………………………………………………………………………

4. Each channel is split into 8 time slots so can be used by 8 phones at the same time. How many mobile phones could Vodaphone support at one time if there was no frequency reuse?

……………………………………………………………………………………………

These questions need to be answered using the OFCOM website that gives details of all mobile phone base stations in the UK (http://www.sitefinder.radio.gov.uk/)

5 (a) How many base stations are there within 200m of your home?

………………………………………………………………………………………………

(b) What is the power of the transmitters?

………………………………………………………………………………………………

6 (a) How many base stations are there within 200m of the college (postcode EC1V 7LA)?

………………………………………………………………………………………………

(b) What is the power of the transmitters?

………………………………………………………………………………………………

7 (a) How many base stations are there within 200m of Addingham (a large village in Yorkshire)?

………………………………………………………………………………………………

(b) What is the power of the transmitters?

……………………………………………………………………………………………..

8. Explain you answers to questions 5, 6 and 7.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..